
Gorman-Redlich ManufacturingPRIVATE 


257 West Union Street


Athens, OH 45701

EAS Encoder/Decoder PC EAS Setup Program

Version 1.5 - 05/18/98


Use with EAS1 V5.9 and above EPROM

1
Introduction -


The EAS Setup Program, EASETUP.EXE, is designed to run on any IBM PC or clone with using either serial port COM1 or COM2.  The program runs as a DOS application on a PC using any version of DOS from 3.1 and up, or in Windows or Windows95 in DOS mode. 


A serial cable with a male DB9 on one end and female DB9 on the other end may be used to connect the male DB9 on the PC port to the female DB9 on the encoder/decoder computer port.  When the encoder/decoder detects the proper code sequences from the PC, it will receive a packet of setup information from the PC and store it in non-volatile memory.  The setup program allows the operator to select the desired setup information from menus before downloading it to the encoder decoder.


To operate properly, the setup program requires the transmitted data to be echoed back by the encoder/decoder.  However the program can be used without connection to the encoder/decoder, if a loopback connector is connected to the PC COM port.  Loopback connectors can be purchased from PC supply stores, loopback connectors are often supplied with PC diagnostic programs such as Norton Utilities, or they may be fabricated from a female DB9 connector  by connecting the following conductors together: 1-4-6-9, 2-3, 7-8.  Pin 5 is not connected.


When requested by the customer, this program will be used by Gorman-Redlich to set up an encoder decoder with the information supplied by the customer on the order form.  In this case, the unit is ready for operation when received by the customer.  The setup information is supplied to the customer on a floppy disk so that the program may be used later by the customer to change or update the setup information.

2
Installation -


The EASETUP program is written to run as a DOS application so that it will be compatible with as many PC type computers as possible.  On newer Windows based systems, simply go to the DOS prompt and continue with these instructions as though it were a DOS system.  The program and associated files are distributed on a high density 3.5" floppy diskette, but may be ordered on other types of diskettes such as a 5.25" diskette, if desired.  It may be transferred  from one type of diskette to another by the user by simply using the DOS COPY command.


The simplest, although probably not the best, form of operation is to run the program directly from the floppy disk.  In this case, insert the diskette into floppy drive A: or B:. Before running directly from the diskette, make a backup copy of the distribution diskette using the DOS copy command.  At the DOS prompt, type A: or B: to correspond to the floppy drive containing the diskette.  The DOS prompt will then show A:> or B:> corresponding to your selection.  At that point type EASETUP and <ENTER> (press the ENTER key), and the program will start (the program name is EASETUP.EXE).  


This form of operation is the only option for older computers with no hard drive.  If a hard drive is available, much faster operation can be obtained by installing the program on the hard drive.  Another advantage is that the data on the floppy drive is protected from unintended change when working from the hard drive.  The installation is very simple.  At the DOS prompt for the hard drive (usually C:>), make a directory for the program and associated files.  If the directory name is to be EAS1, the following command: "MD EAS1 <ENTER>"  (<ENTER> means to just press the ENTER key) will make the directory.  The command "CD EAS1 <ENTER>" (or whatever name was used) will select the new directory.  The command "COPY A:*.* <ENTER>" or "COPY B:*.* <ENTER>" (depending on whether the diskette is in floppy drive A: or B:) will copy the contents of the diskette to the hard drive.  The copy is the extent of the hard drive installation.  The program may be executed at any time by going to the directory ("CD EAS1 <ENTER>") and typing "EASETUP <ENTER>".


The files which are sent on the diskette are:


Filename

Contents


CTY.DEF

List of all states and counties and corresponding FIPS codes.  


EVENT.DEF

A list of all EAS 3 letter event codes and text description or each.


ORG.DEF

A list of the 5 EAS 3 letter originator codes and a text description of each.


EAS.INI

Station Setup Information and PC Installation Information.


CTY.SEL

Counties selected as counties in the station's listening area.


RPSH.SEL

Required preselected header codes selected for the station.


OPSH.SEL

Optional preselected header codes selected for the station.


The DEF files are definition files that do not change with user setup.  These files must be updated as this information changes or expands.  The INI and SEL files contain data that is selected for the station or PC.  These files will change as the setup changes.


When the EAS setup software is started the first time on the PC, run main menu item 6 to enter the PC installation data.  This will select the serial port to be used to download to the encoder/decoder, and it will select time zone information for the PC.  It should be necessary to enter this information only once after the installation.

3
Description -


The setup program allows selection and downloading of the following information:



Station Information -




Station call letters.




Passwords (operator and optional technician).




State Primary.




Station time zone.




Station Daylight Time.




Station originator code.



Encoder/Decoder Setup Information -




Counties in Listening Area - Up to 31 county names and state abbreviation (plus the code for all counties in the state of license)




FIPS codes with corresponding state abbreviations and corresponding state FIPS codes from up to 6 states.




Counties to be sent with required monthly tests.




Counties to be sent with required weekly tests.




Preselected Header Codes - 4 (may be expanded to 8) required pre-selected header codes and priorities.  Up to 20 optional pre-selected header codes and priorities.



Date and Time -




Current UCT date and time from the PC.



The program keeps a copy of your setup on a disk file so that the next time you run the program, you begin with your last setup.  This makes it easy to quickly make minor changes to the setup.

4
Operation -


The program is started from the DOS prompt by typing EASETUP. The first thing that the user will see is the following main menu:

             Gorman‑Redlich EAS1 Encoder/Decoder V 1.5 PC Setup Program



Should be run on EAS1 Encoder/Decoder V 5.9 and later.

                                  MAIN MENU

               1  Enter Encoder/Decoder Setup Information

               2  Send the data over the RS232 port

               3  Reserved for future expansion

               4  Reserved for future expansion

               5  Set the Encoder/Decoder Time to the PC Time

               6  Enter PC installation data

               q  Quit this program and exit to DOS

          Type the number or letter of the menu item to select it


From this main menu the selections are made by entering the corresponding numbers 1 through 6 or q to exit.


4.1
Item 1 - Enter Encoder/Decoder Setup Information



When this Item is selected, the user will be allowed to select all the necessary setup data to allow the encoder/decoder to function properly at that station and in that listening area.  The following subsections will discuss the various items that are to be selected.



4.1.1
Entering Station Setup Information 




In all cases, use the ARROW keys to move the cursor adjacent to the desired entry.  When the cursor is beside the desired entry, strike the ENTER key.  The following sections describe the manner in which each item is entered.  An example of the station setup entry screen follows: 


 Enter Call Letters, Operator password, Technician password


 Select State Primary and Station Time Zone/Daylight Time


 ARROW keys ‑ Position cursor on desired parameter


 ENTER key  ‑ Make the desired selection


 q key      ‑ Exit screen after selection

      
     
WOUB[FM] ‑ Station Call Letters

           

1234 ‑     Operator Password

           

2345 ‑     Technician Password

           

Station is a State Primary

           

Atlantic Time Zone

           

Station does not go on Daylight Time in April

           

PEP ‑      Primary Entry Point System




4.1.1.1
Call Letters - The call letters may be up to 8 characters.  With the exception of '+', '-',  delete character, or control characters (below 20H, 'space' character), any ASCII character may be used in the call letter field.  After selecting call letter entry with the ENTER key, type all the letters of the call.  If it is not 8 characters, finish it with spaces.  After 8 characters and spaces have been entered, the cursor will exit the call letter field.  In case of a mistake, repeat the process.




4.1.1.2
Station Operator Password - A 4 digit password to allow an operator to compose an alert or send an alert.  The password can contain only digits from 0-9.  All 4 digits must be entered.  In case of a mistake, repeat the process.




4.1.1.3
Station Technician Password - A 4 digit password to allow a technician to set modulation levels and change other setup information on the encoder decoder.  This password can be made the same as the operator password if only one password is desired.  The password can contain only digits from 0-9.  All 4 digits must be entered.  In case of a mistake, repeat the process.




4.1.1.4
Station State Primary - Striking the ENTER key with the cursor beside the state primary field will toggle the selection between either "Station is a state primary" or "Station is not a state primary".  Leave the entry field when the correct display is showing.




4.1.1.5
Station Time Zone - Striking the ENTER key with the cursor beside the time zone field will sequentially display each of the following time zones: Atlantic, Eastern, Central, Mountain, Pacific, Alaska, and Hawaii.  Leave the entry field when the correct display is showing. 




4.1.1.6
Station Daylight Time - Striking the ENTER key with the cursor beside the daylight time field will toggle the selection between either "Station goes on Daylight Time in April" or "Station does not go on Daylight Time in April".  Leave the entry field when the correct display is showing. 





4.1.1.7
Station Originator Code - Striking the ENTER key with the cursor beside the originator field will sequentially display each of the following originator codes: EAN, PEP, WXR, CIV, and EAS.  Leave the entry field when the correct display is showing.



4.1.2

seq level3 \h \r0 
Selecting Counties in the Station's Listening Area




After the station information is entered, the county codes must be selected.  The state selection screen will appear to show what states are presently selected as in the following example:


This is county and state selection. You may have up to 6 states.


ARROW keys ‑ Position cursor to desired option


ENTER key  ‑ Select option


DELETE key ‑ Remove the state the cursor is on


q key      ‑ Exit county and state selection

       _ 
Display current selected states and counties

        
Add a new state

         
Edit OH

         
Edit PA

         
Edit WV




From this screen it is possible to: 





1) display the currently selected counties and associated state, 





2) add a new state and selected counties from it, 





3) edit (add or delete counties) for any of the currently selected states.





4) delete the state and all its county selections.




The cursor will initially appear on the line with "Display current selected states and counties".  The cursor may be moved up or down to the other lines to make selections.  Once the cursor is positioned on the line for the desired operation, the <ENTER> key is pressed to make the selection.  In the case of the "Edit" lines, the DELETE key may be pressed to delete the state and its selected counties.  Whenever the state and county selections are satisfactory, pressing the "Q" key will exit the county selection screen.




When the first item is selected, county codes previously selected are displayed on the screen.  This screen may be exited back to the state selection screen by pressing any key.  By using this function, it is easy to determine whether the selections are satisfactory.  If so, the county selection function may be exited with the "Q" key.  The following screen is an example of this display: 



From OH:



009  Athens        045  Fairfield     049  Franklin      053 Gallia



073  Hocking       079  Jackson       089  Licking       105  Meigs



167  Washington



From PA:



001  Adams         003  Allegheny     005  Armstrong     007  Beaver



009  Bedford       001  Berks         013  Blair         015  Bradford



017  Bucks         019  Butler



From WV:



107  Wood




If it is necessary to add counties from a state other than the states shown selected, the first item "Add a new state" must be selected.  In this case, the state abbreviation must first be entered to access the counties for that state (for instance: OH for Ohio).  If it is only necessary to add or delete counties from one of the states already selected, move the cursor down to that state and press <ENTER>.  Whether a new state is being added or an existing state is to be edited, the county selection screen is used.  The following screen is an example of the county selection screen:



Choose from OH. Use arrows, pg‑up/down, <ENT> to select, q to quit

********************************************************************************

*   001  Adams         023  Clark        »045  Fairfield     067  Harrison     *

*   003  Allen         025  Clermont      047  Fayette       069  Henry        *

*   005  Ashland       027  Clinton      »049  Franklin      071  Highland     *

*   007  Ashtabula     029  Columbiana    051  Fulton       »073  Hocking      *

*  »009  Athens        031  Coshocton    »053  Gallia        075  Holmes       *

*   011  Auglaize      033  Crawford      055  Geauga        077  Huron        *

*   013  Belmont       035  Cuyahoga      057  Greene       »079  Jackson      *

*   015  Brown         037  Darke         059  Guernsey      081  Jefferson    *

*   017  Butler        039  Defiance      061  Hamilton      083  Knox         *

*   019  Carroll       041  Delaware      063  Hancock       085  Lake         *

*   021  Champaign     043  Erie          065  Hardin        087  Lawrence     *

********************************************************************************

  9 Athens        OH 45 Fairfield     OH 49 Franklin      OH 53 Gallia        OH

 73 Hocking       OH 79 Jackson       OH  7 Alfalfa       OK> 8 Atoka         OK

  9 Beaver        OK 10 Beckham       OK 11 Adams         NE 12 Antelope      NE

 13 Arthur        NE 14 Banner        NE 15 Brooke        WV 16 Aurora        SD

 17 Entire State  OH

 16 Counties selected from 5 States




When the county selection screen is entered, the first 44 counties for that state will be displayed in alphabetical order.  These counties may be selected or de-selected by moving the cursor with the arrow keys to a county and pressing the ENTER key.  When a county is selected, it will have a >> symbol beside it, and it will be included in the selected county list that is being built in the bottom portion of the screen.  If a county is mistakenly selected, simply move the cursor to the county again, press enter, and the >> symbol will disappear, and it will be removed from the list.  Counties for that state beyond the 44 shown on the display, may be accessed by the use of the page up and page down keys.  The selected county list at the bottom of the screen shows all counties currently selected along with their state abbreviations.  In addition, the prompt at the bottom reports the number of counties and states currently selected.




When all desired counties have been selected, press the "Q" key on the PC to exit back to the state selection screen. The second item, "Display current selected states and counties" may again be selected to review the county selections.  The process of adding or editing states and counties and reviewing selections is continued until the selections are satisfactory, and then the state selection screen is exited by the "Q" key. 



4.1.3
Entering County of License




Upon exiting the selection of counties in the station's listening area, it is possible to enter the county of license.  Since the county of license must be one of the counties in the listening area, this selection must come after selecting the counties in the listening area.  The selection screen will appear as follows:

Select your County of License.

ARROW keys ‑ Position cursor on desired county

ENTER key  ‑ Select or Deselect a county

q key      ‑ Exit screen after selection

  1 Adams         OH  2 Allen         OH  3 Ashland       OH  4 Athens        OH

  5 Vinton        OH> 6 Adair         OK  7 Alfalfa       OK  8 Atoka         OK

  9 Beaver        OK 10 Beckham       OK 11 Adams         NE 12 Antelope      NE

 13 Arthur        NE 14 Banner        NE 15 Brooke        WV 16 Aurora        SD




If a county was previously selected as the county of license, it will be indicated with the >.  If the counties in the listening area have been changed, make sure that the county of license indicated is the one that you want.  When counties in the listening area change, the county of license defaults to the first county in the list.  




Select the county of license in the same manner as the counties in the listening area were selected - by moving the cursor to the desired county and pressing <ENTER>.  The software will allow only one county to be selected, but one county must be selected before exit.  When the desired county has been selected, exit this screen by pressing the "Q" key.



4.1.4
Entering Counties for Required Monthly Tests




Upon exiting the selection of counties in the station's listening area, it is possible to enter the counties to be sent with an outgoing, and checked for an incoming, required monthly test.  Since these counties must be among the counties in the listening area, this selection must come after selecting the counties in the listening area.  The selection screen will appear as follows:

Select counties for Required Monthly Test:

ARROW keys ‑ Position cursor on desired county

ENTER key  ‑ Select or Deselect a county

a key      ‑ Select all counties

q key      ‑ Exit screen after selection

  1 Adams         OH  2 Allen         OH  3 Ashland       OH  4 Athens        OH

> 5 Vinton        OH> 6 Adair         OK> 7 Alfalfa       OK> 8 Atoka         OK

  9 Beaver        OK 10 Beckham       OK 11 Adams         NE 12 Antelope      NE

 13 Arthur        NE 14 Banner        NE 15 Brooke        WV 16 Aurora        SD

 17 Entire State  OK




Any counties previously selected to go out with an RMT will be indicated with the >.  If the counties in the listening area have been changed since the last RMT counties selection, make sure that the counties indicated are the ones that you want.  If counties in the listening area have been changed, RMT county selections revert to the default which is all counties selected.  Select or deselect these counties in the same manner as the counties in the listening area were selected - by moving the cursor to the desired county and pressing <ENTER>.  When the desired counties have been selected, exit this screen by pressing the "Q" key.




The required monthly test counties can include any up to all of the counties in the listening area.  At least one location must be selected in order that a valid RMT header will be sent.  If it is attempted to deselect all locations, the county of license will be automatically selected.




In addition to the individual counties, there is a code available for the entire state of the county of license (in this example, Oklahoma).  If this selection is made, the decoder will respond to the code for all counties in the state of license.  However, the encoder will not include that code with the list of counties in an outgoing header.  To use that code in an outgoing RMT header, the header must be composed rather than using the RMT key.



4.1.5
Entering Counties for Required Weekly Tests




Upon exiting the selection of counties for the required monthly test, it is possible to enter the counties to be sent with an outgoing required weekly test.  Since these counties must be among the counties in the listening area, this selection must come after selecting the counties in the listening area.  The selection screen will appear as follows: 

Select counties for Required Weekly Test:

ARROW keys ‑ Position cursor on desired county

ENTER key  ‑ Select or Deselect a county

a key      ‑ Select all counties

q key      ‑ Exit screen after selection

  1 Adams         OH  2 Allen         OH  3 Ashland       OH  4 Athens        OH

  5 Vinton        OH> 6 Adair         OK  7 Alfalfa       OK  8 Atoka         OK

  9 Beaver        OK 10 Beckham       OK 11 Adams         NE 12 Antelope      NE

 13 Arthur        NE 14 Banner        NE 15 Brooke        WV 16 Aurora        SD




Any counties previously selected to go out with an RWT will be indicated with the >.  If the counties in the listening area have been changed since the last RWT counties selection, make sure that the counties are shown correctly.  If counties in the listening area have been changed, RWT county selections revert to the default which is, only the county of license selected.  Select or deselect these counties in the same manner as the counties in the listening area were selected - by moving the cursor to the desired county and pressing <ENTER>.    When the desired counties have been selected, exit this screen by pressing the "Q" key.




The required weekly test counties can include any up to all of the counties in the listening area.  At least one location must be selected in order that a valid RWT header will be sent.  If it is attempted to deselect all locations, the county of license will be automatically selected.



4.1.6
Entering Pre-selected Header Codes




The four mandatory pre-selected header codes for EAN, EAT, RWT, and RMT, are built-in and do not need to be entered in this step.




Up to 20 optional preselected header codes may be selected.  Each of the pre-selected header codes contains three parts:




1)
Event.




2)
Priority.




3)
Counties in the station's listening area which are affected.




Initially, the preselected header codes are summarized in the main preselected header code screen.  The first 10 preselected header codes are shown initially.  By pressing the PAGE DOWN key, the next 10 will be shown.  Pressing the PAGE UP key will return to the first 10.  An example of this screen is as follows:


Select your Optional Preselected Header Codes.


PAGE keys  - Switch to another 10 preselected header codes


ARROW keys ‑ Position cursor on desired entry


ENTER key  ‑ Edit or add an entry


DELETE key ‑ Delete an entry


q key      ‑ Exit the Optional Preselected Header Code Selection


Entry Event      Priority      


Counties

 
_1    TOA        Send this alert in 10 minutes 
All

 
 2    SVA        Send this alert in 15 minutes 
Some

 
 3    SVS        Send this alert in  5 minutes 
Some

 
 4    FFA        Send this alert in 10 minutes 
None

 
 5    Not Used

 
 6    Not Used

 
 7    Not Used

 
 8    Not Used

 
 9    Not Used


10    Not Used

 
Current Preselected Header Codes




The cursor is initially positioned by the first entry.  It may be moved up and down from entry 1 to entry 10 by using the arrow keys.  The entry selected by the cursor may be deleted by pressing the DELETE key.  When an entry is deleted, "Not Used" will appear in the Event field.




The cursor may be positioned beside a "Not Used" entry and <ENTER> pressed to add a new entry.  Likewise, it may be positioned beside an existing entry and <ENTER> pressed to edit or review an existing entry.  In either case a selection screen for selecting events and priorities and a selection screen for selecting associated counties in the station's listening area will be entered.




The event and priority section screen is as follows:

          Preselected Header Code number:  1. Select both event and priority.


ARROW keys ‑ Position cursor on Event or Priority


ENTER key  ‑ Select an Event or Priority


q key      ‑ Exit screen after selection

                        Select from these events:


>1 TOA  Tornado Watch                   2 TOR  Tornado Warning

  
 3 SVA  Severe Thunderstorm Watch       4 SVR  Severe Thunderstorm Warning

  
 5 SVS  Severe Weather Statement        6 SPS  Special Weather Statement

  
 7 FFA  Flash Flood Watch               8 FFW  Flash Flood Warning

  
 9 FFS  Flash Flood Statement          10 FLA  Flood Watch

 
11 FLW  Flood Warning                  12 FLS  Flood Statement

 
13 WSA  Winter Storm Watch             14 WSW  Winter Storm Warning

 
15 BZW  Blizzard Warning               16 HWA  High Wind Watch

 
17 HWW  High Wind Warning              18 HUA  Hurricane Watch

 
19 HUW  Hurricane Warning              20 HLS  Hurricane Statement

 
21 RWT  Required Weekly Test           22 RMT  Required Monthly Test

 
23 CEM  Civil Emergency Message        24 DMO  Demo Message

 
25 ADR  Administrative Message         26 EVI  Evacuation Immediate


27 TSW  Tsunami Warning                28 TSA  Tsunami Watch

 
29 NPT  National Periodic Test         30 NIC  National Information Center


31 EAN  Emergency Action Notification  32 EAT  Emergency Action Termination


Select from these priorities for pre‑selected header codes:


> 1 Never send this alert               2 Send this alert in 15 minutes

  
  3 Send this alert in 10 minutes       4 Send this alert in 5 minutes

  
  5 Send this alert immediately
   




A selection arrow will appear beside any selections that may have been made previously.  By pressing the arrow keys the cursor may be moved to any event, and <ENTER> pressed to select that event.  Selecting one event deselects any other event  had been selected for this code number.  After selecting the desired event, use the arrow keys to move the cursor down to the priority section at the bottom and select the desired priority in the same manner.  When the selection for this entry is complete, exit by pressing the "Q" key.  This screen cannot be exited until both an event and priority have been selected.




When the event/priority selection screen is exited, the counties affected must be entered in the preselected header code county entry screen.  A screen similar to the following  will appear:



Select counties for Preselected Header Code number:  1

ARROW keys ‑ Position cursor on desired county

ENTER key  ‑ Select or Deselect a county

a key      - Select all counties

q key      ‑ Exit screen after selection

> 1 Athens        OH> 2 Fairfield     OH> 3 Franklin      OH> 4 Gallia        OH 

> 5 Hocking       OH> 6 Jackson       OH> 7 Licking       OH> 8 Meigs         OH

> 9 Washington    OH>10 Adams         PA>11 Allegheny     PA>12 Armstrong     PA

>13 Beaver        PA>14 Bedford       PA>15 Berks         PA>16 Blair         PA

>17 Bradford      PA>18 Bucks         PA>19 Butler        PA>20 Wood          WV




Only the counties in the station's listening area that have previously been selected will be shown.  In this example all counties have been selected.  Any of these counties may be de-selected by moving the cursor to the left of them and pressing <ENTER>.  In addition to selecting or de-selecting individual counties with <ENTER>, pressing "A" will select all counties.  When the desired selections have been made, press "Q" to exit this screen back to the main preselected header code screen.




If the counties in the listening area have been changed since the preselected header codes were last entered, review the existing selections carefully and make changes as necessary.  It is likely that these selections will now be incorrect, if the counties in the listening area were changed.




When all entries are completed, press "Q" in the main preselected header code screen to exit the preselected header code entry section and return to the main menu screen.  The Encoder/Decoder Setup Information entry is now complete.  It may be reviewed and/or changed by again entering main menu item 1.


4.2

seq level2 \h \r0 
Item 2 - Sending the Data over the RS232 port



With the RS232 cable connected to both the PC and the Encoder/Decoder as described in the introduction, the data just entered may be transferred from the PC to the Encoder/Decoder.  Insure that the Encoder/Decoder is powered up and in the normal display with the Decoder Scanning and the Encoder Ready (ie: not in a setup menu, processing an alert, sending a message, etc.), and the RS232 cable is connected from the computer port of the encoder/decoder to the correct PC COM port.  Press the 2 key, and the data will be sent to the Encoder/Decoder.  The data being sent is displayed at the bottom of the PC screen during transfer.  When the transfer is complete, the screen returns to the main menu display.



There are two types of data errors that may occur during attempts to download to the encoder/decoder - timeout errors and echoed data errors.  Each character that is transmitted to the encoder/decoder from the PC is echoed back to the PC by the encoder/decoder.  The EASETUP program then compares the echoed character to the original character, and it reports an error if they are different.  A timeout error occurs when the PC waits longer than a few seconds for the echoed character from the PC.  When either error occurs, the download is aborted, and the software returns to the Main Menu.  Once the cause of the error is found and the error cleared, the download may be attempted again.  Typical causes for the errors are: RS232 cable not connected, encoder/decoder not powered up, encoder/decoder not in main display (ie: in some menu function or otherwise busy), and PC set to wrong COM port.  If no problem is found, re-boot the encoder/decoder, and try again.


4.3
Item 3 - For future expansion 


4.4
Item 4 - For future expansion


4.5
Item 5 - Set the Encoder Time to the PC Time



Pressing 5 will result in the following display:

                   EAS Time Setting Function

                   EST:        12/12/96 19:13:00

                   UCT:        12/13/96 00:13:00

         Press <ENTER> to download time to Encoder/Decoder



The display will show the PC local time and UCT (Universal Coordinated Time or Greenwich Mean Time).  The times are displayed so that the user can check to make sure they are correct.  The difference in hours for UCT - Local Time is: AST=4, EST=5, CST=6, MST=7, PST=8, AST=9, HST=10; ADT=3, EDT=4, CDT=5, MDT=6,  PDT=7, ADT=8, and HDT=9. 



If the PC clock contains the correct local time, and the correct PC time zone and daylight time information has been entered (see next section), then the UCT time will be correct.  The PC local time may be corrected using the standard DOS and/or Windows time setting functions.  On the date that the time change occurs, most PCs will not change the time automatically, and this must be done manually.  However, if this is done and the PC is reading the correct local time, the program will display these two times correctly.



Insure that the Encoder/Decoder is powered up and in the normal display with the Decoder Scanning and the Encoder Ready (ie: not in a setup menu, processing an alert, sending a message, etc.) and the RS232 cable is connected from the computer port of the encoder/decoder to the correct PC COM port.   By pressing <ENTER>, the UCT time will be downloaded to the encoder/decoder.  If the encoder/decoder has been given the correct station time zone and daylight time information (see sections 4.1.1.5 & 4.1.1.6), then the encoder decoder will display the correct local time after this download.



There are two types of data errors that may occur during attempts to download to the encoder/decoder - timeout errors and echoed data errors.  Each character that is transmitted to the encoder/decoder from the PC is echoed back to the PC by the encoder/decoder.  The EASETUP program then compares the echoed character to the original character, and it reports an error if they are different.  A timeout error occurs when the PC waits longer than a few seconds for the echoed character from the PC.  When either error occurs, the download is aborted, and the software returns to the Main Menu.  Once the cause of the error is found and the error cleared, the download may be attempted again.  Typical causes for the errors are: RS232 cable not connected, encoder/decoder not powered up, encoder/decoder not in main display (ie: in some menu function or otherwise busy), and PC set to wrong COM port.  If no problem is found, re-boot the encoder/decoder, and try again.


4.6
Item 6 - PC Installation Data



The serial port to be used in downloading must be selected, if this has not been done in previous sessions.  Selecting item 1 will allow selection of either COM1 or COM2.  This selection should be made based upon which physical PC port to which the DB9 cable will be connected.



The time zone for the PC, and whether the PC time zone changes to daylight time in April, must be entered for the downloaded times to be correct.  Note that the information is not asking whether the PC is currently on daylight time.  It only needs to know whether the PC ever goes on daylight time.  Given this information, it can check as to when it needs to be on daylight time.



The following is an example of the installation screen:


Select PC COM Port, PC Time Zone Daylight Time


ARROW keys ‑ Position cursor on desired parameter


ENTER key  ‑ Make the desired selection


q key      ‑ Exit screen after selection

           
    _
COM1 is the selected PC COM port

           

Central Time Zone

           

PC does not go on Daylight Time in April


4.7
Item q - Exiting to DOS



When finished with the setup program, it may be exited to return to the DOS operating system by pressing the 'q' key.  The familiar DOS '>' prompt will then be seen on the screen.






